Supplementary Materials

These Supplementary Materials support the reproducibility, algorithmic stability, statistical reliability, and real-tree-location input compatibility analyses reported in the main text. This document includes the experimental environment and task configuration, baseline algorithm parameters and fairness settings, key algorithm parameters, parameter sensitivity analysis, convergence curves, supplementary ablation figures, ROS 2/RViz offline route visualization results, execution-oriented proxy assessment, statistical significance tests, and the real-tree-location planning case.

Table S1. Experimental environment and task configuration.

| Parameter | Value |
| --- | --- |
| Orchard area size | 120 m × 120 m |
| Number of citrus trees | 80 |
| Number of obstacles | 11 |
| Obstacle type | Static circular obstacles |
| Number of UGVs | 4 |
| Initial UGV positions | (0,0), (120,0), (0,120), (120,120) |
| Initial battery levels | 100, 80, 60, 40 |
| Observation radius | 2.0 m |
| Safety boundary | 1.5 m |
| Theoretical observation poses per tree | 4 |
| Random seed set | 42–61 |
| Number of repeated runs | 20 |
| Scenario generation method | Matched random instances generated under a unified environment-generation mechanism |

Table S2. Baseline algorithm parameters and fairness settings.

| Algorithm | Population / Search agents | Maximum search budget | Key setting | Fairness note |
| --- | --- | --- | --- | --- |
| F-LCA | — | 1500 | Energy-aware task allocation, multi-view observation mapping, pheromone-guided path construction, and late-stage cooperative restructuring | Same environment generation mechanism, same random seed set, same multi-view observation model, and same evaluation protocol |
| SISR-VRP | — | 1500 | Tree-level nearest-start partition, nearest-neighbor ordering, contiguous string removal, greedy reinsertion, and limited intra-route 2-opt refinement; multi-view observation poses are rendered using the shared observation model during evaluation | Same environment generation mechanism, same random seed set, same tree-level task ownership, same multi-view observation renderer, and same evaluation protocol; no F-LCA-specific residual-energy-aware cooperative task allocation, boundary exchange, layered refinement, or late-stage cooperative restructuring is used |
| ACO | 20 | 1500 | Ant-based candidate search with pheromone update and heuristic transition rule; candidate set size k = 8 | Same random seed set, same instance generation logic, same multi-view observation model, and same metric calculation method |
| GA | 20 | 1500 | Genetic encoding with selection, crossover, and mutation operators | Same random seed set, same instance generation logic, same multi-view observation model, and same metric calculation method |
| PSO | 20 | 1500 | Discrete assignment encoding and priority-based ordering update | Same random seed set, same instance generation logic, same multi-view observation model, and same metric calculation method |

Note: F-LCA and SISR-VRP do not use conventional population-size parameters; therefore, “—” is reported in the Population / Search agents column. All algorithms were run on 20 matched random instances with random seeds ranging from 42 to 61. Runtime was reported only as an engineering computational-cost indicator and was not used as the sole criterion for fairness. It should be noted that the unified search budget in this study refers to the maximum number of outer search iterations or route-reconstruction attempts under the internal search structure of each algorithm, and does not indicate that different algorithms have exactly the same number of basic objective-function evaluations. Because F-LCA, SISR-VRP, and the swarm-intelligence algorithms have different internal search structures, fair comparison in this study was based on the same problem instances, the same observation task definition, the same random seed set, and the same evaluation protocol. Runtime was reported separately as an additional engineering-cost indicator.

Table S3. Key algorithm parameter settings.

| Parameter category | Parameter name | Value |
| --- | --- | --- |
| General setting | Random seed set | 42–61 |
| General setting | Number of repeated runs | 20 |
| General setting | Maximum search budget | 1500 |
| General setting | Orchard area size | 120 m × 120 m |
| General setting | Number of citrus trees | 80 |
| General setting | Number of obstacles | 11 |
| General setting | Number of UGVs | 4 |
| General setting | Initial battery levels | 100, 80, 60, 40 |
| General setting | Observation radius | 2.0 m |
| General setting | Safety boundary | 1.5 m |
| General setting | Theoretical observation poses per tree | 4 |
| F-LCA | Basic candidate set size | 10 |
| F-LCA | Late-stage candidate set size | 9 |
| F-LCA | Number of path candidates | 4 |
| F-LCA | Pheromone weight α | 1.00 → 1.20 |
| F-LCA | Heuristic weight β | 2.80 → 3.80 |
| F-LCA | Pheromone evaporation coefficient | 0.94 → 0.90 |
| F-LCA | Path balance weight | 0.68 |
| F-LCA | Obstacle penalty weight | 0.19 |
| F-LCA | Missing observation pose penalty | 18.0 |
| F-LCA | Incomplete tree penalty | 22.0 |
| F-LCA | Energy protection term | 18.0 |
| F-LCA | Local search frequency | Every 20 iterations |
| F-LCA | Late-stage local search frequency | Every 12 iterations |
| F-LCA | Minimum / expected observation poses per tree | 4 / 4 |
| SISR-VRP | Initial route construction | Nearest-start partition + nearest-neighbor ordering |
| SISR-VRP | Minimum string removal length | 2 |
| SISR-VRP | Maximum string removal length | 5 |
| SISR-VRP | Number of removed strings per iteration | 1–2 |
| SISR-VRP | Removal bias | Expensive route segment / long route / random route |
| SISR-VRP | Removal bias probability | 0.40 / 0.30 / 0.30 |
| SISR-VRP | Repair strategy | Greedy insertion |
| SISR-VRP | Candidate insertion positions | All positions if route length ≤ 12; otherwise route endpoints and up to 8 sampled intermediate positions |
| SISR-VRP | Load-balance insertion penalty coefficient | 3.0 |
| SISR-VRP | Local refinement | Limited intra-route 2-opt |
| SISR-VRP | Local refinement frequency | Every 100 iterations |
| SISR-VRP | Maximum 2-opt trials per refinement | 8 |
| SISR-VRP | Acceptance rule | Accept improving moves only |
| GA | Population size | 20 |
| GA | Number of generations | 1500 |
| PSO | Number of particles | 20 |
| PSO | Number of iterations | 1500 |
| PSO | Inertia weight w | 0.72 |
| PSO | Learning factors c1, c2 | 1.45, 1.45 |
| ACO | Number of ants | 20 |
| ACO | Number of iterations | 1500 |
| ACO | Candidate set size k | 8 |
| ACO | Pheromone weight α | 1.0 |
| ACO | Heuristic weight β | 2.0 |
| ACO | Pheromone evaporation coefficient | 0.90 |

Note: SISR-VRP was used as a tree-level VRP-oriented ruin-and-recreate baseline. It first performs vehicle assignment and visiting-sequence reconstruction at the tree-task-unit level, and then generates observation-viewpoint routes through the same multi-view observation rendering and evaluation procedure used for F-LCA. This baseline uses only string removal, greedy reinsertion, and limited 2-opt local refinement. It does not include the residual-energy-aware cooperative task allocation, region boundary correction, boundary exchange, layered refinement, or late-stage cooperative restructuring mechanisms of F-LCA. Except for scenario parameters, the remaining algorithm parameters were determined through preliminary experiments and kept unchanged across all 20 random-seed experiments. No instance-specific parameter tuning was performed.

Fig. S1. Convergence comparison of the four iterative algorithms under the unified search budget.

Note: (a) Average historical best Primary Cost convergence curves of the four iterative swarm-intelligence algorithms; (b) enlarged convergence curves of the average historical best Primary Cost for F-LCA and ACO. The results are based on 20 random seeds. SISR-VRP adopts a ruin-and-recreate search process based on string removal and reinsertion, and its search trajectory is not directly comparable with the population-iterative historical best sequences of F-LCA, ACO, GA, and PSO. Therefore, SISR-VRP was not included in this convergence-curve comparison. Its final performance was reported together with the other algorithms in Fig. 3 of the main text, Supplementary Table S7, Supplementary Table S5a, and Supplementary Table S5b.

| (a) | (b) |
| --- | --- |

Fig. S2. Comprehensive performance comparison of different ablated variants.

Note: (a) Comparison of mean Total Cost; (b) comparison of mean Obstacle Penalty. The results are based on 20 repeated experiments with random seeds.

Fig. S3. Boxplot of Primary Cost distributions for different ablated variants.

Note: The boxplot is based on 20 repeated experiments with random seeds. The boxes indicate interquartile ranges, and the horizontal lines inside the boxes indicate medians.

Fig. S4. Ablation results for multi-view observation completeness.

Note: (a) Comparison of Effective Coverage; (b) comparison of mean Primary Cost.

Supplementary Algorithm S1. Pseudocode of the late-stage cooperative restructuring mechanism.

| Line | Operation |
| --- | --- |
| Input | Current vehicle-task sets A = {A1, A2, ..., Am}, route set P = {P1, P2, ..., Pm}, visiting-order set R = {R1, R2, ..., Rm}, multi-view observation poses Q, obstacle set O, battery levels E, current metric vector M, maximum search budget T, stagnation thresholds, and cooldown intervals. |
| Output | Restructured vehicle-task sets A*, routes P*, and visiting orders R*. |
| 1 | Initialize A* ← A, P* ← P, and R* ← R. |
| 2 | At each search iteration, evaluate total distance, obstacle penalty, energy-load ratio, primary cost, and reference total cost for all UGVs. |
| 3 | If the energy-load gap or task-load gap exceeds the preset threshold, perform adaptive region correction. |
| 3.1 | Select the UGV with the largest energy-load ratio as the donor vehicle and the UGV with the smallest energy-load ratio as the receiver vehicle. |
| 3.2 | Rank candidate trees in the donor task set according to boundary distance, transfer cost, and local task complexity. |
| 3.3 | Move a candidate boundary tree to the receiver only if the transfer preserves multi-view coverage and reduces load imbalance or path cost. |
| 3.4 | Repair the updated task partition to avoid excessive task concentration in any vehicle. |
| 4 | If the search stagnates and the boundary-exchange cooldown condition is satisfied, perform boundary exchange. |
| 4.1 | Sort donor candidates by decreasing energy-load ratio and receiver candidates by increasing energy-load ratio. |
| 4.2 | Select a boundary tree from the donor task set. |
| 4.3 | Remove the selected tree from the donor visiting order and insert it into the receiver visiting order at the position with the lowest local route cost. |
| 4.4 | Apply local visiting-order refinement to both donor and receiver routes. |
| 4.5 | Recompute the primary cost, reference total cost, obstacle penalty, total distance, and energy range of the candidate solution. |
| 4.6 | Accept the exchange if the candidate solution improves the layered evaluation key, or if it maintains the primary cost within the relaxation margin while reducing the energy-range indicator. |
| 5 | If the layered-refinement trigger condition is satisfied, perform progressive refinement. |
| 5.1 | Save the current task sets, routes, and visiting orders. |
| 5.2 | Transfer selected boundary trees from overloaded vehicles to underloaded vehicles. |
| 5.3 | Rebuild the affected UGV routes and refine their local visiting orders. |
| 5.4 | Accept the refined solution if it improves the layered evaluation key or reduces the energy-range indicator under the primary-cost relaxation condition; otherwise, roll back to the saved solution. |
| 6 | If long-term stagnation persists and a historical best solution is available, perform destroy-and-repair. |
| 6.1 | Remove a short contiguous segment from the historical visiting order of a selected vehicle. |
| 6.2 | Reinsert the removed trees into positions with lower local route cost. |
| 6.3 | Apply local visiting-order refinement and accept the reconstructed solution only if it improves the current evaluation metric. |
| 7 | Whenever a restructuring operation is accepted, update A*, P*, R*, and the best recorded metric vector. |
| 8 | Repeat Steps 2–7 until the maximum search budget or termination condition is reached. |
| 9 | Return A*, P*, and R*. |

Note: The layered evaluation key prioritizes observation completeness, incomplete-tree ratio, energy feasibility, Primary Cost, Energy Range, and reference Total Cost in sequence. Therefore, the late-stage restructuring procedure improves route organization without relaxing the predefined multi-view observation requirement.

Fig. S5. ROS 2/RViz-based visualization of F-LCA offline cooperative observation routes.

Note: The figure shows a representative visualization of F-LCA offline cooperative observation routes in the ROS 2/RViz environment. Routes with different colors correspond to different vehicles, and gray circular regions represent obstacles. This figure is used to illustrate that the offline planned routes can be further mapped to a ROS 2/RViz visualization environment. It does not indicate that real closed-loop UGV control or field deployment validation has been completed in this study.

Definitions of execution-oriented proxy metrics

To further explain the execution friendliness of offline planned routes under a unified post-processing protocol, several proxy metrics were defined, including execution trajectory length, estimated execution time, minimum obstacle-entity clearance, minimum safety clearance, and turning-constraint violation ratio.

The smoothed execution trajectory of the k-th UGV is defined as:

|  | Pkexe=pk,1exe,pk,2exe,…,pk,nkexe | (S1) |
| --- | --- | --- |

The execution trajectory length of the k-th UGV is then defined as:

|  | Lkexe=r=1nk-1pk,r+1exe-pk,rexe2 | (S2) |
| --- | --- | --- |

Given the average vehicle speed, dwell time at each observation point, and the number of actually visited observation viewpoints, the simplified estimated execution time is calculated as:

|  | Tkexe=Lkexev+nkobstobs | (S3) |
| --- | --- | --- |

Given the center position, physical radius, and safety-boundary width of the j-th obstacle, the minimum obstacle-entity clearance and minimum safety clearance of the k-th UGV are defined as:

|  | Ck,minsafe=minp∈Pkexedobsp-dsafe | (S4) |
| --- | --- | --- |

Furthermore, let the local equivalent turning radius be calculated from three consecutive trajectory points. When this radius is lower than the minimum turning-radius threshold, the corresponding segment is counted as a turning-constraint violation. The turning-constraint violation ratio is defined as:

|  | Vkturn=q=2nk-1IRk,qRminnk-2×100% | (S5) |
| --- | --- | --- |

where I(⋅) is the indicator function, which takes the value of 1 when the condition is satisfied and 0 otherwise. The Turn Violation Ratio / % reported in Table S4 represents the proportion of trajectory segments that violate the turning constraint, rather than the absolute number of violations. The above execution-oriented proxy metrics are used only as supplementary evaluation metrics to illustrate the potential of further mapping offline planning results to execution-layer analysis under a unified post-processing protocol. They are not incorporated into the main optimization objective and are not used as the basis for judging the main performance ranking of the algorithms.

Table S4. Execution-oriented proxy assessment results under a unified post-processing protocol.

| Algorithm | Execution Path Length / m | Estimated Execution Time / s | Minimum Obstacle Clearance / m | Minimum Safety Clearance / m | Turn Violation Ratio / % |
| --- | --- | --- | --- | --- | --- |
| F-LCA | 1433.84 ± 46.37 | 2752.30 ± 57.96 | 0.964 ± 0.120 | -0.536 ± 0.120 | 12.67 ± 2.18 |
| SISR-VRP | 1554.04 ± 94.78 | 2902.55 ± 118.47 | 0.983 ± 0.078 | -0.517 ± 0.078 | 11.22 ± 2.15 |
| ACO | 1877.20 ± 148.60 | 3306.51 ± 185.75 | 0.854 ± 0.556 | -0.646 ± 0.556 | 10.48 ± 1.56 |

Note: Results are reported as mean ± standard deviation over 20 repeated experiments with random seeds. Minimum Obstacle Clearance denotes the minimum clearance between the trajectory and the physical obstacle boundary, whereas Minimum Safety Clearance denotes the clearance margin relative to the predefined safety boundary. Because Minimum Safety Clearance is calculated relative to the safety boundary rather than the physical obstacle boundary, its value can be negative. A negative value indicates that the trajectory enters the predefined safety buffer, but it does not indicate collision with the physical obstacle entity. Turn Violation Ratio denotes the proportion of trajectory segments whose local turning radius is lower than the minimum allowable turning radius. F-LCA, SISR-VRP, and ACO were selected for this assessment to compare the proposed method, a strong VRP-oriented baseline, and a classical ant-colony path-planning baseline under the unified post-processing protocol. This assessment does not constitute a complete execution-layer comparison of all baseline algorithms. It is used only to illustrate the potential of further mapping offline planned routes to execution-layer analysis and is not used as the basis for the main performance comparison among the five algorithms.

Table S5a. Friedman test results.

| Metric | Algorithms included | Number of paired instances | Friedman test result | Interpretation |
| --- | --- | --- | --- | --- |
| Primary Cost | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | p < 0.001 | Significant overall difference among algorithms |
| Total Cost | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | p < 0.001 | Significant overall difference among algorithms |
| Total Distance | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | p < 0.001 | Significant overall difference among algorithms |
| Obstacle Penalty | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | p < 0.001 | Significant overall difference among algorithms |
| Energy Range | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | p < 0.001 | Significant overall difference among algorithms |
| Runtime | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | p < 0.001 | Significant overall difference among algorithms |
| Theoretical Coverage | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | Not tested | All algorithms achieved 100.00 ± 0.00% |
| Effective Coverage | F-LCA, SISR-VRP, ACO, GA, PSO | 20 | Not tested | All algorithms achieved 100.00 ± 0.00% |

Note: The Friedman test was conducted based on 20 matched random instances. Because all five algorithms achieved 100.00 ± 0.00% in both Theoretical Coverage and Effective Coverage, significance tests were not conducted for the coverage metrics.

Table S5b. Wilcoxon-Holm paired test results using F-LCA as the reference.

| Metric | Pairwise comparison | Direction of difference | Wilcoxon test result after Holm correction | Interpretation |
| --- | --- | --- | --- | --- |
| Primary Cost | F-LCA vs SISR-VRP | F-LCA lower | p < 0.001 | F-LCA significantly improves the core objective over SISR-VRP |
| Primary Cost | F-LCA vs ACO | F-LCA lower | p < 0.001 | F-LCA significantly improves the core objective over ACO |
| Primary Cost | F-LCA vs GA | F-LCA lower | p < 0.001 | F-LCA significantly improves the core objective over GA |
| Primary Cost | F-LCA vs PSO | F-LCA lower | p < 0.001 | F-LCA significantly improves the core objective over PSO |
| Total Distance | F-LCA vs SISR-VRP | F-LCA lower | p < 0.001 | F-LCA generates shorter cooperative observation paths than SISR-VRP |
| Total Distance | F-LCA vs ACO | F-LCA lower | p < 0.001 | F-LCA generates shorter paths than ACO |
| Total Distance | F-LCA vs GA | F-LCA lower | p < 0.001 | F-LCA generates shorter paths than GA |
| Total Distance | F-LCA vs PSO | F-LCA lower | p < 0.001 | F-LCA generates shorter paths than PSO |
| Obstacle Penalty | F-LCA vs SISR-VRP | SISR-VRP lower | p < 0.001 | SISR-VRP shows lower obstacle penalty than F-LCA |
| Obstacle Penalty | F-LCA vs ACO | F-LCA lower | p < 0.001 | F-LCA significantly reduces obstacle penalty compared with ACO |
| Obstacle Penalty | F-LCA vs GA | F-LCA lower | p < 0.001 | F-LCA significantly reduces obstacle penalty compared with GA |
| Obstacle Penalty | F-LCA vs PSO | F-LCA lower | p < 0.001 | F-LCA significantly reduces obstacle penalty compared with PSO |
| Energy Range | F-LCA vs SISR-VRP | SISR-VRP lower | p < 0.001 | SISR-VRP shows better load-balance indicator under this metric |
| Energy Range | F-LCA vs ACO | F-LCA lower | p < 0.001 | F-LCA shows lower energy range than ACO |
| Energy Range | F-LCA vs GA | GA lower | p < 0.001 | GA shows lower energy range but with much longer paths and higher obstacle penalty |
| Energy Range | F-LCA vs PSO | PSO lower | p < 0.001 | PSO shows lower energy range but with much longer paths and higher obstacle penalty |
| Runtime | F-LCA vs SISR-VRP | SISR-VRP lower | p < 0.001 | SISR-VRP has lower computational time |
| Runtime | F-LCA vs ACO | F-LCA lower | p < 0.001 | F-LCA is faster than ACO |
| Runtime | F-LCA vs GA | GA lower | p < 0.05 | GA has lower computational time |
| Runtime | F-LCA vs PSO | PSO lower | p < 0.05 | PSO has lower computational time |

Note: The Wilcoxon signed-rank test was conducted using the 20 matched random instances as paired samples, and the Holm method was used for multiple-comparison correction. Primary Cost and Total Distance are the core metrics used in this study to evaluate route-organization quality under complete multi-view observation constraints. Obstacle Penalty, Energy Range, and Runtime are auxiliary metrics used to explain the trade-offs among obstacle cost, load balancing, and computational cost. SISR-VRP performed better in Obstacle Penalty, Energy Range, and Runtime, whereas F-LCA maintained advantages in Primary Cost and Total Distance. The complete Wilcoxon-Holm paired test results, including the Total Cost metric, are provided in the data repository as wilcoxon_holm_results.csv. This table lists the main pairwise comparison metrics discussed in the manuscript.

Overall, the statistical results show that F-LCA maintained advantages over SISR-VRP and the three classical baseline algorithms in the two core route-organization metrics, namely Primary Cost and Total Distance. Because all five algorithms achieved 100.00 ± 0.00% in both Theoretical Coverage and Effective Coverage, the coverage metrics were not further subjected to significance testing, but were treated as prerequisite conditions indicating whether the complete observation constraint was satisfied. Meanwhile, SISR-VRP performed better in Obstacle Penalty, Energy Range, and Runtime, indicating that its VRP-oriented route-restructuring strategy based on string removal and reinsertion has strong local connection optimization capability and computational efficiency. Therefore, the comparison between F-LCA and SISR-VRP should be interpreted as a trade-off between route compactness and local route-restructuring efficiency under complete multi-view observation constraints.

Table S6. Data and planning-result summary of the real-tree-location case.

| Item | Value / Description |
| --- | --- |
| Source of tree locations | KML-exported citrus tree markers |
| Number of real tree locations | 43 |
| Coordinate system before conversion | WGS84 longitude-latitude |
| Coordinate system after conversion | Local planar x-y coordinates |
| Planning region size | 62.43 m × 50.45 m |
| Number of UGVs | 4 |
| Theoretical coverage | 100.00% |
| Effective coverage | 100.00% |
| Total path length | 588.88 m |
| Energy range | 5.91% |
| Output files | real_citrus_tree_points_xy.csv; real_orchard_best_path.json; real_orchard_case_metrics.csv |

Note: This real-tree-location case was used to verify the compatibility of F-LCA with real spatial tree-location inputs and the feasibility of the workflow from KML tree-location points to local planar coordinates and then to cooperative observation route generation. This case was not used as an additional multi-algorithm statistical comparison experiment and does not constitute real closed-loop UGV deployment validation.

Table S7. Performance comparison of the five algorithms under the unified environment.

| Algorithm | Primary Cost | Total Cost | Total Distance / m | Theoretical Coverage / % | Effective Coverage / % | Obstacle Penalty | Energy Range / % | Runtime / s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F-LCA | 1413.76 ± 72.88 | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 100.00 ± 0.00 | 100.00 ± 0.00 | 418.85 ± 192.70 | 14.27 ± 3.31 | 86.42 ± 1.23 |
| SISR-VRP | 1535.19 ± 91.29 | 1818.11 ± 416.37 | 1487.43 ± 91.21 | 100.00 ± 0.00 | 100.00 ± 0.00 | 251.37 ± 124.76 | 3.49 ± 4.13 | 21.17 ± 4.42 |
| ACO | 2024.11 ± 177.68 | 4137.78 ± 492.35 | 1754.35 ± 96.47 | 100.00 ± 0.00 | 100.00 ± 0.00 | 1419.75 ± 543.76 | 20.18 ± 6.05 | 433.48 ± 130.70 |
| GA | 3771.39 ± 250.84 | 4064.03 ± 267.24 | 3330.73 ± 194.30 | 100.00 ± 0.00 | 100.00 ± 0.00 | 2319.27 ± 492.89 | 0.65 ± 0.17 | 62.53 ± 30.93 |
| PSO | 6624.41 ± 396.82 | 7115.08 ± 424.58 | 5185.52 ± 254.46 | 100.00 ± 0.00 | 100.00 ± 0.00 | 7573.13 ± 1026.67 | 0.63 ± 0.37 | 75.75 ± 43.86 |

Note: Results are reported as mean ± standard deviation over 20 repeated experiments with random seeds.

Parameter sensitivity analysis

| (a) | (b) |
| --- | --- |
| (c) |  |

Fig. S6. Sensitivity analysis of F-LCA under key parameter perturbations.

Note: (a) Primary Cost; (b) Total Distance; (c) Obstacle Penalty. Blue bars indicate the perturbation results of the obstacle-penalty weight λo, and gray-blue bars indicate the perturbation results of the residual-energy-aware task allocation exponent partition_battery_power. Under the same multiplier, the left bar represents the perturbation result of λo, and the right bar represents the perturbation result of partition_battery_power. All results are based on 20 matched random instances. The multiplier 1.0× indicates the baseline parameter setting used in the main experiment. The complete parameter sensitivity results are provided in Table S8.

The results show that F-LCA maintained 100.00% Theoretical Coverage and Effective Coverage under ±20% perturbations of key parameters, indicating that the generated multi-view observation routes were stable under moderate parameter changes. Changes in the obstacle-penalty weight mainly affected the trade-off between obstacle risk and path compactness, whereas changes in the residual-energy-aware task allocation exponent mainly affected initial task-region organization and the distribution of path costs. In contrast, moderate changes in the path-balancing weight λb and the observation-completeness penalty multiplier λc did not change the final route structure. Specifically, λb mainly changed the contribution of the balancing term to Total Cost, whereas λc did not trigger additional coverage penalties once the complete observation constraint was already satisfied. The complete numerical summary is provided in Table S8.

Table S8. Sensitivity analysis results for key F-LCA parameters.

| Parameter | Multiplier | Actual value |  | Primary Cost | Total Cost | Total Distance / m | Obstacle Penalty | Energy Range / % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| λo | 0.8× | 0.152 | 1397.38 ± 69.09 |  | 2424.70 ± 354.46 | 1333.14 ± 56.87 | 422.60 ± 174.57 | 13.98 ± 3.64 |
| λo | 0.9× | 0.171 | 1408.61 ± 54.79 |  | 2404.49 ± 163.87 | 1331.23 ± 45.36 | 452.51 ± 181.80 | 13.72 ± 2.26 |
| λo | 1.0× | 0.190 | 1413.76 ± 72.88 |  | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λo | 1.1× | 0.209 | 1424.34 ± 69.23 |  | 2455.40 ± 282.18 | 1342.72 ± 54.65 | 390.51 ± 161.21 | 13.95 ± 3.07 |
| λo | 1.2× | 0.228 | 1424.10 ± 69.51 |  | 2452.72 ± 243.60 | 1337.43 ± 56.84 | 380.13 ± 165.15 | 14.06 ± 2.70 |
| pbat | 0.8× | 0.680 | 1424.05 ± 58.82 |  | 2572.09 ± 292.81 | 1346.01 ± 57.75 | 410.74 ± 154.33 | 15.47 ± 2.75 |
| pbat | 0.9× | 0.765 | 1427.17 ± 75.34 |  | 2530.09 ± 306.79 | 1341.05 ± 54.66 | 453.27 ± 212.29 | 14.76 ± 3.22 |
| pbat | 1.0× | 0.850 | 1413.76 ± 72.88 |  | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| pbat | 1.1× | 0.935 | 1419.18 ± 53.70 |  | 2498.59 ± 290.31 | 1338.19 ± 39.39 | 426.30 ± 142.95 | 14.51 ± 3.28 |
| pbat | 1.2× | 1.020 | 1401.00 ± 70.59 |  | 2440.67 ± 275.53 | 1325.19 ± 63.13 | 399.01 ± 190.59 | 14.24 ± 3.39 |
| λb | 0.8× | 0.544 | 1413.76 ± 72.88 |  | 2441.46 ± 327.67 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λb | 0.9× | 0.612 | 1413.76 ± 72.88 |  | 2453.90 ± 328.15 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λb | 1.0× | 0.680 | 1413.76 ± 72.88 |  | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λb | 1.1× | 0.748 | 1413.76 ± 72.88 |  | 2478.76 ± 329.13 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λb | 1.2× | 0.816 | 1413.76 ± 72.88 |  | 2491.20 ± 329.62 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |

Table S8. Continued.

| Parameter | Multiplier | Actual value | Primary Cost | Total Cost | Total Distance / m | Obstacle Penalty | Energy Range / % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| λc | 0.8× | 14.4 / 17.6 | 1413.76 ± 72.88 | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λc | 0.9× | 16.2 / 19.8 | 1413.76 ± 72.88 | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λc | 1.0× | 18.0 / 22.0 | 1413.76 ± 72.88 | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λc | 1.1× | 19.8 / 24.2 | 1413.76 ± 72.88 | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |
| λc | 1.2× | 21.6 / 26.4 | 1413.76 ± 72.88 | 2466.33 ± 328.64 | 1334.18 ± 59.11 | 418.85 ± 192.70 | 14.27 ± 3.31 |

Note: Results are reported as mean ± standard deviation over 20 matched random instances. λo denotes the obstacle-penalty weight, λb denotes the path-balancing weight, pbat denotes the residual-energy-aware task allocation exponent partition_battery_power, and λc denotes the observation-completeness penalty multiplier. For λc, the two values in the Actual value column denote the missing observation pose penalty and the incomplete tree penalty, respectively. Theoretical Coverage and Effective Coverage were 100.00 ± 0.00% under all parameter perturbation settings. The multiplier 1.0× indicates the baseline parameter setting used in the main experiment.